Tutorial 4

Compiling Pixel ROD Slave Firmware and generating bitfiles with ISE locally

March 2022

- Download the correct branch from git repo  (fw_Separated2 shown here)

git clone --single-branch -b fw_Separated2 https://gitlab.cern.ch/atlas-pixel/daq/pixelrod_firmware/RodSlave

Open the ISE project - Rodslave/Firmware/Pixel/ISE

The .xise ise projects are located here.

Since Spartan6 FPGAs are used, ISE Design Suite has to be used. In this tutorial we are using the slave firmware for calibration mode called pixel_rodSlave_calibration.xise . Upon first opening the project you may get a prompt called Locate Missing Source FIles. This will be solved in the next steps.

- Source git hash

In order to generate the gitIDPkg.vhd you have to run the command "source hash2id.sh" from the Firmware/ directory (use git bash if on a windows machine)

source hash2id.sh

- Regenerate cores

Regenerate all cores in the ISE project. This will get rid of the pop up.

- Check design properties

Ensure that it matches as below

- system.xmp

In order to do this step, the axi_ethernet license is required. This license does not come with any full ISE or Vivado license options and has to be purchased separately. This is used for the microblaze

https://www.xilinx.com/support/documentation/ip_documentation/axi_ethernet/v3_01_a/ds759_axi_ethernet.pdf

Double click system_i.xmp

Platform studio will open, click on generate netlist there

Note that the path to the system.xmp has no spaces, if there are spaces there will be an error because XPS will not be able to read the file.

- Synthesize, implement, generate bitstream

Select rodSlave_top_calibration and go through the synthesize, implement and generate bitstream steps. The warnings can be ignored.

The generated bitfile will appear in the folder of the. xise project.

In order to load this file in the modules at SR1, it has to be converted to .ace file which will be in another tutorial